## TDA9384/TDA9363 Model

## Color TV Service Manual Catalogue

- Technology Specification and Feature…..…………………………………Page 1

- The survey……………………………………………………………Page 3

- The main chips instruction…………………………………………. Page 5

- Signal process……………………………………………………… Page 10

- The horizontal and vertical circuit…………………………………Page 11

- Power supply……………………………………………………….. Page 12

- Appendix1(Factory menu)……………….………………………….Page13

- Appendix 2(IC introduction) …...…………………………………..Page 16

- Technology Specification and Feature

I2C Bus Control

#### Auto Search

#### Off/On Timer

Teletext

Rated Voltage: 	     AC 110V-250V    50Hz/60Hz

Power Consumption:	  150W

Sound output:      	  6W×2

Screen size				  Picture tube 29/34 inch

System:                Color		              Sound

RF    PAL SECAM          D.K / B.G/I

AV    NTSC

Channel coverage:	    VHF Low channel (VL)	=48.25 to 147.25 MHz

VHF high channel (VH)	=154.25 to 463.25 MHz

UHF channel U)		    =471.25 to 863.25 MHz

#### IF                  38.9MHz

External input/output	 2sets Video/Audio(L/R) input

YCbCr input

SVHS input

Video/Audio(L/R) output

Notice: The main difference of TDA9363 and TDA9384 is TDA9363 has build-in TELETEXT function.  But there is no difference on the pin descriptions.

2. The survey

Table 1, the main IC and functions

| Number | Function | IC | Remark |
| --- | --- | --- | --- |
| 1 | Power supply | KA5Q1265RF | N801 |
| 2 | CPU + SIGNAL PROCESSER | TDA9384/TDA9363 | #### N201 |
| 3 | Field driver | TDA8351 | N301 |
| 4 | Sound processor | TDA9859 | N1001 |
| 5 | Sound driver | TDA7495S | N601 |
| 6 | AV SWITCH | TC4052BP | N1002 (Video) N1003 (Audio) |
| 7 | Port expanding | HEF4094 | N502 |
| 8 | IF amplifier | HX0021D | N101 |
| 9 | Memory | AT24C08 | N202 |
| 10 | Photo-coupler | HS817B | N802 |
| 11 | IR-receiver | HS0038A | NA01 |
| 12 | 8V Regulator | L7808CV | N804 |
| 13 | 5V Regulator | L7805CV | N805 N703 |

Fig1. TDA9384/TDA9363 MODE COLOR TV illustration

3. The main chips instruction

The UOC (“Ultimate One Chip”) TDA9363 is adopted in this chassis. This IC is the first available component that contains the complete control and small signal functionality needed for a TV application in one device.

Ⅰ, The UOC TDA9384 pins function description：（total 64 pins）

Pin1: Standby control,“1” is on,“0”is off.

Pin2: SCL.

Pin3: SDA.

Pin4: Tuning PWM output.

Pin5: Auto AV control SW, connected with the SCART2’s 8th pin.

Input. The rising edge or the falling edge operates.

Pin6: Key board input.

Pin7: Mute control,“1” is mute,“0”is off.

Pin8: CTL, the earth magnetic field rectification output.

Pin9: Pin12、Pin18、Pin30、Pin35、Pin41、Pin55: GND.

Pin10: LED, the lamp control output.

“1” is on standby, the lamp is light,

“0” is turn-on，the lamp is dim.

Pin11: RELAY, control the K701,“1”is degaussing,“0”is not.

Pin13: SECAM PLL, connected with a capacitance.

Pin14: +8V power source supply.

Pin15: Using a capacitor of 220n in series to GND, This pin decouples the

internal digital supply voltage of the video processor and

minimizes the disturbance to the sensitive analogue parts.

Pin16: PHI-2 control loop, this pin requires a capacitor at 2.2nF (C) in

series to GND.

Pin17: PHI-1 control loop, the loop filter connected to pin 17 is suitable for

various signal conditions like strong/weak and VCR signal. This is

achieved by switching of the loop filter time constant by

changing the PHI-1 output current.

Pin18: GND.

Pin19: Bandgap decoupling, the bandgap circuit provides a very stable

and temperature independent reference voltage. This reference

voltage (4.0 V) ensures optimal performance of the analogue video

processor part of the TDA9363 and is used in almost all functional

circuit blocks.

Pin20: East-west pillow signal output.

Pin21: Pin22：Vertical drive output.

Pin23: Pin24：IF input。

Pin25: Reference current, This pin requires a resistor to ground. The

optimal reference current is 100mA which is determined by

this resistor. The 100mA reference current should not be changed

because the geometry processor is optimised for this current.

Furthermore the output current of vertical drive and EW are

proportional to this current.

Pin26: Vertical sawtooth, This pin requires a capacitor to ground of 100nF

Pin27: AGC output. This output is used to control (reduce) the tuner gain

for strong RF signals.

Pin28: Audio de-emphasis.

Pin29: Sound decoupling. This pin requires a capacitor connected to

ground. The pin acts as a low pass filter needed for the DC

feedback loop.

Pin30: GND.

Pin31: Sound loop filter.

Pin32: AVL filter.

Pin33: Horizontal drive signal output, needs a resistor in series to +8V.

Pin34: Sandcastle output/flyback input,

Pin35: External audio input, this pin should be grounded in this chassis.

Pin36: EHT tracking/ overvoltage protection. If something is wrong, the

anode high voltage rises, the heater voltage will rise too. When the

rising voltage arrive some limit, the V406 works, the voltage of pin

36 will exceed 3.9V, the TDA9363 will stop working.

Pin37: PLL loop filter.

Pin38: CVBS output. Monitor or RF videos can be selected.

Pin39: +8V supply source.

Pin40: CVBS input

Pin42: Y signal input.

Pin43: C signal input.

Pin44: Main audio output, this pin is connected to the TDA9859.

Pin45: RGB signal input blanking.

Pin46, Pin47, Pin48: RGB signal input.

Pin49: ABL. It means beem current limiter input. The R410 is the control resister.

Pin50: Black current input from the CRT board.

Pin51, Pin52, Pin53: RGB drive signal output to the CRT board.

Pin54: +3.3V.

Pin55: GND.

Pin56: +3.3V.

Pin57, Pin58, Pin59: 12MHz crystal。

Pin60: Reset, NC in this chassis.

Pin61: +3.3V

Pin62: NC.

Pin63: This pin is connected to the HEF4094, Functions expanding.

Pin64: IR signal input.

| PIN | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| 1 | P1.3/T1 | port 1.3 or Counter/Timer 1 input |
| 2 | P1.6/SCL | port 1.6 or I2C-bus clock line |
| 3 | P1.7/SDA | port 1.7 or I2C-bus data line |
| 4 | P2.0/TPWM | port 2.0 or Tuning PWM output |
| 5 | P3.0/ADC0 | port 3.0 or ADC0 input |
| 6 | P3.1/ADC1 | port 3.1 or ADC1 input |
| 7 | P3.2/ADC2 | port 3.2 or ADC2 input |
| 8 | P3.3/ADC3 | port 3.3 or ADC3 input |
| 9 | VSSC/P | digital ground for m-Controller core and periphery |
| 10 | P0.5 | port 0.5 (8 mA current sinking capability for direct drive of LEDs) |
| 11 | P0.6 | port 0.6 (8 mA current sinking capability for direct drive of LEDs) |
| 12 | VSSA | analog ground of Teletext decoder and digital ground of TV-processor |
| 13 | SECPLL | SECAM PLL decoupling |
| 14 | VP2 | 2nd supply voltage TV-processor (+8V) |
| 15 | DECDIG | decoupling digital supply of TV-processor |
| 16 | PH2LF | phase-2 filter |
| 17 | PH1LF | phase-1 filter |
| 18 | GND3 | ground 3 for TV-processor |
| 19 | DECBG | bandgap decoupling |
| 20 | AVL/EWD (1) | Automatic Volume Levelling /East-West drive output |
| 21 | VDRB | vertical drive B output |
| 22 | VDRA | vertical drive A output |
| 23 | IFIN1 | IF input 1 |
| 24 | IFIN2 | IF input 2 |
| 25 | IREF | reference current input |
| 26 | VSC | vertical sawtooth capacitor |
| 27 | TUNERAGC | tuner AGC output |
| 28 | AUDEEM/SIFIN1(1) | audio deemphasis or SIF input 1 |
| 29 | DECSDEM/SIFIN2(1) | decoupling sound demodulator or SIF input 2 |
| 30 | GND2 | ground 2 for TV processor |
| 31 | SNDPLL/SIFAGC(1) | narrow band PLL filter /AGC sound IF |
| 32 | AVL/SNDIF/REF0/AMOUT(1) | Automatic Volume Levelling / sound IF input / subcarrier reference output /AM output(non controlled) |
| 33 | HOUT | horizontal output |
| 34 | FBISO | flyback input/sandcastle output |
| 35 | AUDEXT/QSSO/AMOUT(1) | external audio input /QSS intercarrier out /AM audio output (non controlled) |
| 36 | EHTO | EHT/overvoltage protection input |
| 37 | PLLIF | IF-PLL loop filter |
| 38 | IFVO/SVO | IF video output / selected CVBS output |
| 39 | VP1 | main supply voltage TV-processor (+8 V) |
| 40 | CVBSINT | internal CVBS input |
| 41 | GND1 | ground 1 for TV-processor |
| 42 | CVBS/Y | external CVBS/Y input |
| 43 | CHROMA | chrominance input (SVHS) |
| 44 | AUDOUT /AMOUT(1) | audio output /AM audio output (volume controlled) |
| 45 | INSSW2 | 2nd RGB / YUV insertion input |
| 46 | R2/VIN | 2nd R input / V (R-Y) input |
| 47 | G2/YIN | 2nd G input / Y input |
| 48 | B2/UIN | 2nd B input / U (B-Y) input |
| 49 | BCLIN | beam current limiter input/V-guard input |
| 50 | BLKIN | black current input |
| 51 | RO | Red output |
| 52 | GO | Green output |
| 53 | BO | Blue output |
| 54 | VDDA | analog supply of Teletext decoder and digital supply of TV-processor (3.3 V) |
| 55 | VPE | OTP Programming Voltage |
| 56 | VDDC | digital supply to core (3.3 V) |
| 57 | OSCGND | oscillator ground supply |
| 58 | XTALIN | crystal oscillator input |
| 59 | XTALOUT | crystal oscillator output |
| 60 | RESET | reset |
| 61 | VDDP | digital supply to periphery (+3.3 V) |
| 62 | P1.0/INT1 | port 1.0 or external interrupt 1 input |
| 63 | P1.1/T0 | port 1.1 or Counter/Timer 0 input |
| 64 | P1.2/INT0 | port 1.2 or external interrupt 0 input |

Ⅱ, Memory AT24C08 is an E2PROM of 8k, pins describe as follows:

Pin1, Pin2, Pin4, Pin7: GND.

Pin3, Pin8: +5V-1 supply.

Pin5: SDA.

Pin6: SCL.

Ⅲ, HEF4094B is described as follows:（total 16 pins）

Pin1: Connected with UOC Pin63.

Pin2: SDA.

Pin3: SCL.

Pin4, Pin5: AV control switch output, connected to the HEF4052.  data as follows：

| HEF4094B Pin5 #### HEF4052 Pin9 | HEF4094B Pin4 #### HEF4052 Pin10 | STATE |
| --- | --- | --- |
| 0 | 0 | AV1 |
| 1 | 0 | AV2 |
| 0 | 1 | YCbCr |
| 1 | 1 | SVHS |

Pin8: GND.

Pin11, Pin12: Tuner bands control output，data as follows:

| HEF4094B Pin11 | ### HEF4094B Pin12 | BAND |
| --- | --- | --- |
| 0 | 0 | VHF－L |
| 1 | 0 | VHF－H |
| 0 | 1 | UHF |

Pin14: NC.

Pin15: Control port, it is high level in this chassis.

Pin16: +5V supply.

4. Signal process

The main chip is N201 TDA9363, AV control switch HEF4052, sound process chip is TDA9859, sound driver is N601 TDA7495S。

The TV signal inputs into the tuner (U101) from CABLE or antenna. The pin 11 and pin 12 of the N502 are combined to select the band. The pin 4 of the N201 outputs the PWM tuning signal. The IF video signal comes from the IF pin of the tuner. The 38.9MHz IF signal is coupled to the N101 (pre-amplify) and then to SAWF (Z101). After processed in the SAWF, the 38.9MHz signal gets to the pin 23 and pin 24 of TDA9363. The IF circuit in TDA9363 includes such unit as the AGC amplifying circuit, 38.9MHz oscillator, PLL video demodulator, video amplifier, IF identify circuit and AFT circuit. The demodulated signal (CVBS) comes from the pin 38 of TDA9363, the sound signal comes from the pin 44.

The internal CVBS signal needs norm identification then outputs from the pin 38 of TDA9363, via the trap-wave circuit (composed of the V504, Z501, Z502, Z503, V506 and so on) feeds back to the pin 40 of TDA9363. The RGB signal comes from the pin51, Pin52, Pin53 of TDA9363, and outputs to the CRT board.

The V901, V902 and V903 are the R、G、B drive transistors. The V904, V905, V906, V907, V908, V909 are the auto low bright balance level output circuit, and generate the low bright level current into the pin 50 of TDA9363.

The internal sound signal comes from the pin 44 of TDA9363, via the coupling capacitor C224 connects to the pin 3 and 5 of TDA9859. The TDA9859 is the audio effect processor, the TDA9495S is the driver. The TDA9859 includes bass, treble, balance, surround, effect shortcut options.

5. The horizontal and vertical circuit

Through Synchronous separating circuit, the video signal is divided into horizontal-Synchronizing signal and Vertical-Synchronizing signal. The horizontal-Sync pulse coming from the pin 33 is transferred to the horizontal-drive transistor, and will be used to drive the horizontal-transformer. The horizontal-switch transistor is V405, it and the +B supply drives the flyback transformer to generate the anode high voltage, the focus voltage, the screen voltage, the CRT board drive voltage 190V, the vertical drive voltage 17V and 54V.

The East-west pillow signal comes from the pin 20 of TDA9363. The wave is amplified to modulate the horizontal drive circuit. The amplifying circuit composed of the V401, V403, V402 and other peripheral parts.

The vertical sawtooth wave is generated on the pin 21 and 22, and then enters the vertical output amplifier circuit. The vertical output amplifier circuit is realized with the power amplifier IC –TDA8351.

The TDA8351 is a 9 pins vertical deflection circuit (3 Amperes) for DC-coupled 90° or 110°deflection systems with frame frequencies from 50 up to 120 Hz. Only a single supply voltage for the scan and a second supply for the flyback are needed.

The vertical drive currents of TDA9363 pins 21 and 22 are connected to input pins 1 and 2 of the TDA8351. The currents are converted into a voltage by resistor R305. Pin 2 is on a fixed DC level (internal bias voltage, about 2.3V) and on pin 1 the drive voltage can be measured (typical 1.4 Vpp). The outputs (pins 4 and 7) are connected to the series connection of the vertical deflection coil and feedback resistor R306 and R308. The voltage across R306 and R308 is fed via pin 9 to obtain a deflection current which is proportional to the drive voltage. The supply voltage for the TDA8351 is 17V at pin 3. The flyback generator has a separate supply voltage of 54V on pin 6. On pin 8 a vertical guard signal is available. For HF loop stability a damping resistor R309 is connected across the deflection coil.

6. Power supply

The IC of KA5Q1265RF is adapted in this chassis; it is the product of Fairchild. It supplies four DC voltages, one is the +B＝130V, another is Hcc＝25V, the third is 27V (the sound drive voltage), the fourth is +16V. The +16V can generate the +8V, +5V and +3.3V by the special generators.

Fig.2 The power supplies illustration

C811

N801

T801

N802

VD812

+25V To V712, T402, V404

L701

L703

VD705

VD706

VD707

VD708

AC 220V          DC

VD813

+27V To N601

300V

VD811

+16V To V711

+12V

VD810

VD401

+190V To CRT board

T401

VD402

VD403

+17V To Pin 3 of N301

+54V To Pin 6 of N301

N804

N805

N803

N703

+8V To N101, N201, N1001/2/3, CRT board

+5V

+5V-1

+3.3V To N201

To N202, N502, NA01

VD830

7. Appendix 1 – Factory menu

Press  MUTE, CALL, 9, until “9- -”appears on the screen, then input “8”, “0” key to get into factory mode.  “M” will appear on the screen.

Press Menu key to select the menu

Press CH+/CH- to select items, and press V+/V- to adjust the value.

| MENU 1 | Value | Description | M |
| --- | --- | --- | --- |
| PARA5/6 (50H/60H) | 31 | Parallelogram |  |
| BOW5/6 (50H/60H) | 31 | Bow |  |
| HPS5/6 (50H/60H) | 31 | Horizontal center |  |
| WID5/6 (50H/60H) | 31 | Width |  |
| DPC5/6 (50H/60H) | 31 | Parabola |  |
| UCNR5/6 (50H/60H) | 31 | Up-corner |  |
| LCNR5/6 (50H/60H) | 31 | Low-corner |  |
| KEY5/6 (50H/60H) | 31 | Trapezium |  |
| MENU 2 | Value | Description | M |
| VLIN5/6 (50H/60H) | 31 | Vertical linearity |  |
| HIT5/6 (50H/60H) | 31 | Vertical amplitude |  |
| VSC5/6 (50H/60H) | 31 | Vertical S-correction |  |
| VP50/60 (50H/60H) | 31 | Vertical center |  |
| OSDV5/6 (50H/60H) | 31 | OSD vertical position |  |
| VX | 30 | Vertical amplify |  |
| RCUT | 31 |  |  |
| GCUT | 31 |  |  |
| MENU 3 | Value | Description | M |
| RDRV | 31 |  |  |
| GDRV | 31 |  |  |
| BDRV | 31 |  |  |
| WBBRI | 31 |  |  |
| WBCON | 60 |  |  |
| LANG | 4 | OK language option |  |
| YDFP | 7 | Y-delay: SECAM/PAL/NTSC individually |  |
| AGC | 28 |  |  |
| MENU 4 | Value | Description | M |
| VOL | 20 | TDA9859 volume |  |
| 9874 | 10 | TDA9874AH volume |  |
| IFFS | 2 | IF option |  |
| HDOL | 8 | Cathode inspect level |  |
| AGCS | 1 | AGC operate speed |  |
| VG2B | 39 | Screen adjust state |  |
| TILT | 31 |  |  |
| VX16 | 2 | Vertical amplify in 16:9 state |  |

| MENU 5 | Value | Description | M |
| --- | --- | --- | --- |
| OSDL | 0 | OSD bright |  |
| BCF | 2 | Start time in M state |  |
| OKBCF | 4 | Start time in OK state |  |
| BRI50/60 (50H/60H) | 39 | Sub-bright |  |
| CON50/60 (50H/60H) | 61 | Sub-contrast |  |
| COL50/60 (50H/60H) | 61 | Sub-color |  |
| SHA50/60 (50H/60H) | 31 | Sub-sharpness |  |
| HUE50/60 (50H/60H) | 50 | Sub-tint |  |
| MENU 6 | Value | Description | M |
| OPT1 | 204 | Refer to Table 3 |  |
| OPT2 | 3 | Refer to Table 3 |  |
| OPT3 | 115 | Refer to Table 3 |  |
| OPT4 | 243 | Refer to Table 3 |  |
| OPT5 | 55 | Refer to Table 3 |  |
| OPT6 | 127 | Refer to Table 3 |  |
| INIT |  | Reset |  |

M legend explain:

	: Manufacture adjusting item

	: Soft relative items

M state entrance process:

quickly step by step press the key mute, call and 980.

Table 3, the option of menu 6

OP_DEFAULT_OPT_1		0x098 	/*	VG2 mode				No

Comb filter				No

Super woofer				No

AV3						Yes

SVHS					Yes

DVD					No

RGB					No

OSO					Yes */

OP_DEFAULT_OPT_2		0x003	/*	AVL						Yes

Auto Sound				Yes

National Option table 1		No

National Option table 2		No

National Option table 2		No

Function 1				No

Function 2				No

FSL						No */

OP_DEFAULT_OPT_3		0x0F3	/*	AFC_SAVING			Yes

NV_8598					Yes

CHARGING				No

SECAM_SVM			No

Sound DK				Yes

Sound BG				Yes

Sound I					Yes

Sound M					Yes */

OP_DEFAULT_OPT_4		0x0FB	/*	FMWS					Yes

Direct switch on			Yes

AKB					No

LOGO					Yes

HCO					Yes

IDENT_SENSITIVE		Yes

STABLE					Yes

DFL						Yes */

OP_DEFAULT_OPT_5		0x03F	/*	TILT					Yes

NICAM					Yes

BOX					Yes											        		MONITOR				Yes

CALENDAR				Yes												        	GAME   				Yes

RESERVED				No			                	                        ZOOM 16:9 OSVE			No	 */

OP_DEFAULT_OPT_6		0x07F	/*	ENGLISH				YES

SPANISH				YES

ARABRIC				YES												        	GERMAN				YES

FRENCH					YES											        		ROMANIAN				YES

TURKISH				YES

RUSSIAN				NO	 */

OP_DEFAULT_OPT_7		0x00D			/*	OP_FARSI				YES

OP_MALAYSIA			no

OP_ITALIAN			    YES

OP_HUNGARY           YES

OP_MAGNETIC

OP_HOTEL				NO

OP_AGCL1				NO

OP_AGCL2				NO */

OP_DEFAULT_OPT_8     0x004           /*  OP_BBSW               NO

OP_BLACK_BACK       NO

OP_CURTAIN           YES

8. Appendix 2

Fig. 3 HEF4052B illustration

Fig. 4 .1 TDA9859 illustration 1

| 1 | SCINL | SCART input; left channel |
| --- | --- | --- |
| 2 | P1 | port 1 output |
| 3 | MINL | MAIN input; left channel |
| 4 | CSMO | smoothing capacitor of reference voltage |
| 5 | MINR | MAIN input; right channel |
| 6 | VP | supply voltage |
| 7 | SCOUTR | SCART output; right channel |
| 8 | GND | ground |
| 9 | MOUTR | MAIN output; right channel |
| 10 | LINR | input to right loudspeaker channel |
| 11 | CBR1 | bass capacitor connection 1 right channel |
| 12 | CBR2 | bass capacitor connection 2;right channel |
| 13 | n.c. | not connected |
| 14 | CTR | treble capacitor connection;right channel |
| 15 | LOUTR | loudspeaker output; right channel |
| 16 | SCL | serial clock input; I2C-bus |
| 17 | SDA | serial data input/output; I2C-bus |
| 18 | LOUTL | loudspeaker output; left channel |
| 19 | CTL | treble capacitor connection; |
| 20 | n.c. | left channel not connected |
| 21 | CBL2 | bass capacitor connection 2;left channel |
| 22 | CBL1 | bass capacitor connection 1;left channel |
| 23 | LINL | input to left loudspeaker channel |
| 24 | MOUTL | MAIN output; left channel |
| 25 | MAD | module address select input |
| 26 | SCOUTL | SCART output; left channel |
| 27 | CPS2 | pseudo stereo capacitor 2 |
| 28 | AINL | AUX input; left channel |
| 29 | CPS1 | pseudo stereo capacitor 1 |
| 30 | AINR | AUX input; right channel |
| 31 | P2 | port 2 output |
| 32 | SCINR | SCART input signal RIGHT |

Fig. 4.2 TDA9859 illustration 2

Fig. 5.1 TDA8351 illustration 1

Fig. 5.2 TDA8351 illustration 2

FUNCTIONAL DESCRIPTION

The vertical driver circuit is a bridge configuration. The

deflection coil is connected between the output amplifiers,

which are driven in phase opposition. An external resistor

(RM) connected in series with the deflection coil provides

internal feedback information. The differential input circuit

is voltage driven. The input circuit has been adapted to

enable it to be used with the TDA9150, TDA9151B,

TDA9160A, TDA9162, TDA8366 and TDA8376 which

deliver symmetrical current signals. An external resistor

(RCON) connected between the differential input

determines the output current through the deflection coil.

The relationship between the differential input current and

the output current is defined by: Idiff RCON = Icoil RM.

The output current is adjustable from 0.5 A (p-p) to 3 A

(p-p) by varying RM. The maximum input differential

voltage is 1.8 V. In the application it is recommended that

Vdiff = 1.5 V (typ). This is recommended because of the

spread of input current and the spread in the value of

RCON.

The flyback voltage is determined by an additional supply

voltage VFB. The principle of operating with two supply

voltages (class G) makes it possible to fix the supply

voltage VP optimum for the scan voltage and the second

supply voltage VFB optimum for the flyback voltage. Using

this method, very high efficiency is achieved.

The supply voltage VFB is almost totally available as

flyback voltage across the coil, this being possible due to

the absence of a decoupling capacitor (not necessary, due

to the bridge configuration). The output circuit is fully

protected against the following:

thermal protection

short-circuit protection of the output pins (pins 4 and 7)

short-circuit of the output pins to VP.

A guard circuit VO(guard) is provided. The guard circuit is

activated at the following conditions:

during flyback

during short-circuit of the coil and during short-circuit of

the output pins (pins 4 and 7) to VP or ground

during open loop

when the thermal protection is activated.

This signal can be used for blanking the picture tube

screen.

Fig. 7 KA5Q1265RF illustration