Methods of common telecontrol’s bus entry

- channel 30

- channel 88

- minimum volume

- mute

enter into M pattern

1:table 1:

| state after bus entry |  |  |  |
| --- | --- | --- | --- |
| press digital key of 1,2,3,5,6 to indicate |  | press digital key of 4 to indicate |  |
| item | data | item | data |
| VP50 | 06 | BRTS | 1D |
| HIT50 | 15 | press digital key of 7 to indicate |  |
| HPS60 | 14 | item | data |
| VP60 | 02 | POWER | 083 |
| HIT60 | 17 | BUS LINE | OK |
| VLI50 | 0C | BUS CONT | OK |
| VSC50 | 00 | SYNC | NG |
| VLI60 | 0A | press digital key of 8 to indicate |  |
| VSC60 | 00 | OSD | 10 |
| SBY | 08 | press menu key to alternatively indicate M and TEST |  |
| SRY | 08 |  |  |
| RAGC | 24 |  |  |
| RCUT | 20 |  |  |
| GCUT | 75 |  |  |
| BCUT | 1A |  |  |
| GDRV | 31 |  |  |
| BDRV | 36 |  |  |
| COLC | 45 |  |  |
| TNTC | 36 |  |  |
| COLP | 10 |  |  |
| COLS | 40 |  |  |
| SCNT | 0F |  |  |
| HPS50 | 0E |  |  |

Factory telecontrol may enter into more common items

| No.. not USE(s/w) | Symbol |  | Description |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | OSD |  | OSD Horizontal Position |  | 0C |  |
| 2 |  | OPT |  | Optional Setting |  | E3 |  |
| 3 |  | RCUT |  | R CUT OFF |  | 25 |  |
| 4 |  | GCUT |  | G CUT OFF |  | 53 |  |
| 5 |  | BCUT |  | B CUT OFF |  | 67 |  |
| 6 |  | GDRV |  | G DRIVE |  | 67 |  |
| 7 |  | BDRV |  | B DRIVE |  | 3A |  |
| 8 |  | CNTX |  | CONTRAST MAX. |  | 7F |  |
| 9 |  | BRTC |  | BRIGHT CENTER |  | 32 |  |
| 10 |  | COLC |  | COLOR CENTER NTSC |  | 45 |  |
| 11 |  | TNTC |  | TINT CENTER |  | 35 |  |
| 12 |  | COLP |  | COLOR CENTER PAL(difference from COLC) |  | 03 |  |
| 13 |  | COLS |  | COLOR CENTER SECAM |  | 4D |  |
| 14 |  | SCOL |  | SUB COLOR |  | 04 |  |
| 15 |  | SCNT |  | SUB CONTRAST |  | 0F |  |
| 16 |  | CNTC |  | CONTRAST CENTER |  | 20 |  |
| 17 |  | CNTN |  | CONTRAST MIN. |  | 00 |  |
| 18 |  | BRTX |  | BRIGHT MAX.(difference from center) |  | 18 |  |
| 19 |  | BRTN |  | BRIGHT MIN.(difference from center) |  | 10 |  |
| 20 |  | COLX |  | COLOR MAX.(difference from center) |  | 7F |  |
| 21 |  | COLN |  | COLOR MIN. |  | 00 |  |
| 22 |  | TNTX |  | TINT MAX.(difference from center) |  | 28 |  |
| 23 |  | TNTN |  | TINT MIN.(difference from center) |  | 28 |  |
| 24 |  | ST3 |  | SHARP CENTER 3.58NTSC TV |  | 25 |  |
| 25 |  | SV3 |  | SHARP CENTER 3.58NTSC VIDEO |  | 30 |  |
| 26 |  | ST4 |  | SHARP CENTER OTHER TV |  | 25 |  |
| 27 |  | SV4 |  | SHARP CENTER OTHER VIDEO |  | 25 |  |
| 28 |  | SVD |  | SHARP CENTER DVD |  | 25 |  |
| 29 |  | ASSH |  | ASYMMETRY-SHARPNESS |  | 00 |  |
| 30 |  | SHPX |  | SHARP MAX.(difference from center) |  | 1A |  |
| 31 |  | SHPN |  | SHARP MIN.(difference from center) |  | 1A |  |
| 32 |  | TXCX |  | TEXT RGB CONTRAST MAX. |  | 1F |  |
| 33 |  | RGCN |  | TEXT RGB CONTRAST MIN. |  | 00 |  |
| 34 |  | ABL |  | ABL data in detail |  | 37 |  |
| 35 |  | DCBS |  | A part of Video data in detail |  | 33 |  |
| 36 |  | CLTO |  | The data when TV mode & SOUND SYS != M |  | 48 |  |
| 37 |  | CLTM |  | The data when TV mode & SOUND SYS = M |  | 48 |  |
| 38 |  | CLVO |  | The data when YUV mode & SOUND SYS != M |  | 40 |  |
| 39 |  | CLVD |  | The data when YUV mode & SOUND SYS = M |  | 42 |  |
| 40 |  | DEF |  | A part of DEF COMP data in detail |  | 01 |  |
| 41 |  | AKB |  | AKB SYSTEM |  | 00 |  |
| 42 |  | SECD |  | SECAM MODE |  | 0C |  |
| 43 |  | HPS50 |  | Horizontal Position |  | 0D |  |
| 44 |  | VP50 |  | Vertical Position 50Hz |  | 04 |  |
| 45 |  | HIT50 |  | Hight 50Hz |  | 0F |  |
| 46 |  | HPS60 |  | Horizontal Position 60Hz |  | 14 |  |
| 47 |  | VP60 |  | Vertical Position 60Hz |  | 00 |  |
| 48 |  | HIT60 |  | Hight 60Hz |  | 14 |  |
| 49 |  | VLI50 |  | V Linearity 50Hz |  | 1C |  |
| 50 |  | VSC50 |  | VS Correction 50Hz |  | 00 |  |
| 51 |  | VLI60 |  | V Linearity 60Hz |  | 0B |  |
| 52 |  | VSC60 |  | VS Correction 60Hz |  | 04 |  |
| 53 |  | SBY |  | SECAM B-Y BLACK ADJUST |  | 08 |  |
| 54 |  | SRY |  | SECAM R-Y BLACK ADJUST |  | 08 |  |
| 55 |  | BRTS |  | SUB BRIGHT |  | 10 |  |
| 56 |  | RAGC |  | RF AGC |  | 24 |  |
| 57 |  | HAFC |  | AFC GAIN |  | 04 |  |
|  | 58 |  | V25 |  | VOLUME 25 |  | 3D |
|  | 59 |  | V50 |  | VOLUME 50 |  | 50 |
|  | 60 |  | V100 |  | VOLUME 100 |  | 5D |
|  | 61 |  | MUTT |  | Y-MUTE FOR SOFT START |  | 00 |
|  | 62 |  | STAT |  | CONTRAST UP FOR SOFT START |  | 00 |
|  | 63 |  | FLG0 |  | FLAGS for IF |  | 54 |
|  | 64 |  | FLG1 |  | FLAGS |  | 06 |
|  | 65 |  | REFP |  | REF Pulse Positon |  | 00 |
|  | 66 |  | RSNS |  | R SENS |  | 28 |
|  | 67 |  | GSNS |  | G SENS |  | 30 |
|  | 68 |  | BSNS |  | B SENS |  | 2D |
|  | 69 |  | MOD |  | MODE |  | 40 |
|  | 70 |  | STBY |  | VCD / IF STANBY |  | 00 |
|  | 71 |  | SVM |  | SVM |  | 00 |
|  | 72 |  | VBLK |  | V BLK Start / Stop |  | 00 |
|  | 73 |  | VCEN |  | V CENTERING |  | 3F |
|  | 74 |  | HSIZ |  | H SIZE |  | 20 |
|  | 75 |  | PRBR |  | PARABOLA |  | 20 |
|  | 76 |  | TRUM |  | TRAPZIUM |  | 20 |
|  | 77 |  | ECCT |  | EW-CORNER CORRECTION(TOP) |  | 10 |
|  | 78 |  | ECCB |  | EW-CORNER CORRECTION(BOTTOM) |  | 10 |
|  | 79 |  | EHT |  | V EHT / H EHT |  | 04 |
|  | 80 |  | UCOM |  | Miciom Control,Chroma APC Setting |  | 00 |
|  | 81 |  | PYNX |  | normal H.SYNC max |  | 28 |
|  | 82 |  | PYNN |  | normal H.SYNC min |  | 18 |
|  | 83 |  | PYXS |  | search H.SYNC max |  | 22 |
|  | 84 |  | PYNS |  | search H.SYNC min |  | 1E |
|  | 85 |  | RCUTS |  | FOR YCbCr R cutoff(difference from RCUT) |  | 0B |
|  | 86 |  | GCUTS |  | FOR YCbCr G cutoff(difference from GCUT) |  | 00 |
|  | 87 |  | BCUTS |  | FOR YCbCr B cutoff(difference from BCUT) |  | 0A |
|  | 88 |  | GDRVS |  | FOR YCbCr G drive(difference from GDRV) |  | 00 |
|  | 89 |  | BDRVS |  | FOR YCbCr B drive(difference from BDRV) |  | 00 |
|  | 90 |  | NOIS |  | HAFC CONTROL |  | 01 |
|  | 91 |  | AV OPT |  | ＡＶ INPUT FUNCTION CHANGE |  | 01 |
|  | 92 |  | OPT2 |  | Option2 |  | 02 |
|  | 319E1385 |  | AUSTP |  |  |  | 31 |
|  | 94 |  | MODE0 |  |  |  | E1 |
|  | 95 |  | MODE1 |  |  |  | 85 |
|  | 96 |  | OSDF |  | OSD PLL DATA |  | 63 |
|  | 97 |  | PWR |  | Not used in this version |  | 53 |
|  | 98 |  | BUS |  | Bus short status |  | 00 |
|  | 99 |  | MEM |  | Not used in this version |  | 02 |
|  | 100 |  | WB CONTRAST |  |  |  | 64 |
|  | 101 |  | WB BRIGHT |  |  |  | 3C |
|  | 102 |  | WB COLOR |  |  |  | 00 |
|  | 103 |  | MODE3 |  |  |  | D0 |
|  | No |  | Symbol. not USE(s/w) | Description |  |  |
|  | 104 |  | DOWN2 |  |  |  | 87 |
|  | 105 |  | AREA |  |  |  | F7 |
|  | 106 |  | AREA2 |  |  |  | 1F |
|  | 107 |  | CENT |  |  |  | C0 |
|  | 108 |  | DOWN |  |  |  | 98 |
|  | 109 |  | MCOLOR |  |  |  | 46 |
|  | 110 |  | PCOLOR |  |  |  | 52 |
|  | 111 |  | TCOLOR |  |  |  | 1F |
|  | 112 |  | HTVOLUME |  |  |  | 00 |

Above mentioned data are only for your reference. Except that data in table 1 may be adjusted according to different machines, all other parameter data shall not be adjusted unless any necessary.